5x7 Display Experiments Page 9

EXPERIMENT-23   
This experiment shows how to display a feature (such as an animation) AFTER a button has been pressed. 
Press button "A" and SPLASH will show once: 
To test the de-bounce feature, press and release the button very slowly. Notice that the animation only appears once. This is because the button must be released for 40h loops before it is recognised again.  Try 20h loops (or less) and the animation will occur twice on some occasions. This proves how fast the micro is executing the program - it is performing 20h loops of the program in the time the button is released and the micro picks up the "bounce-pulses" at the end of the button-release to show SPLASH the second time.

|  | Experiment-23 for "5x7 Display" Project ;PIC16F84 and only F84 chip ;Press button A for SPLASH! - with debounce Start ORG 0x00 BSF 03,5 ;Go to page1 for setting-up the ports MOVLW 04h ;Put 04 into W MOVWF 05h ; to make RA2 input MOVLW 00h ;Put 00 into W MOVWF 06h ;and make port 6 (port B) all output BCF 03,5 ;Go to page0 for programming GOTO Main Cell1 MOVLW 00h ;place the 5 values into the 5 MOVWF 11h ; files to produce the first cell MOVLW 00h ; as shown above. MOVWF 12h MOVLW 08h MOVWF 13h MOVLW 00h MOVWF 14h MOVLW 00h MOVWF 15h GOTO Scan Cell2 MOVLW 00h ;place the 5 values into the 5 MOVWF 11h ; files to produce the second cell MOVLW 1Ch ; as shown above. MOVWF 12h MOVLW 14h MOVWF 13h MOVLW 1Ch MOVWF 14h MOVLW 00h MOVWF 15h GOTO Scan Cell3 MOVLW 3Eh ;place the 5 values into the 5 MOVWF 11h ; files to produce the third cell MOVLW 22h ; as shown above. MOVWF 12h MOVLW 22h MOVWF 13h MOVLW 22h MOVWF 14h MOVLW 3Eh MOVWF 15h GOTO Scan Cell4 MOVLW 41h ;place the 5 values into the 5 MOVWF 11h ; files to produce the fourth cell MOVLW 41h ; as shown above. MOVWF 12h MOVLW 41h MOVWF 13h MOVLW 41h MOVWF 14h MOVWF 15h GOTO Scan Cell5 MOVLW 00h ;place the 5 values into the 5 MOVWF 11h ; files to produce the fifth cell MOVLW 00h ; as shown above. MOVWF 12h MOVLW 00h MOVWF 13h MOVLW 00h MOVWF 14h MOVLW 00h MOVWF 15h GOTO Scan Scan BSF 05,1 ;Reset 4017 NOP BCF 05,1 MOVF 11h,0 ;Move file 11 into W MOVWF 06h CALL DelD MOVF 12h,0 ;Move file 12 into W MOVWF 06h CALL DelD MOVF 13h,0 ;Move file 13 into W MOVWF 06h CALL DelD MOVF 14h,0 ;Move file 14 into W MOVWF 06h CALL DelD MOVF 15h,0 ;Move file 15 into W MOVWF 06h CALL DelD RETURN DelD DECFSZ 1Bh,1 GOTO DelD MOVLW 00h ;Zero port B to prevent mirroring MOVWF 06 Clk BSF 05,0 ;Clock the 4017 NOP BCF 05,0 RETURN Splash MOVLW 50h ;Each 10h represents 100mS MOVWF 0Ch Splash1 CALL Cell1 ;Display Cell 1 for 1/2 sec DECFSZ 0Ch GOTO Splash1 MOVLW 10h ;Each 10h represents 100mS MOVWF 0Ch Splash2 CALL Cell2 ;Display Cell 2 for 100mS DECFSZ 0Ch GOTO Splash2 MOVLW 10h ;Each 10h represents 100mS MOVWF 0Ch Splash3 CALL Cell3 ;Display Cell 3 for 100mS DECFSZ 0Ch GOTO Splash3 MOVLW 10h ;Each 10h represents 100mS MOVWF 0Ch Splash4 CALL Cell4 ;Display Cell 4 for 100mS DECFSZ 0Ch GOTO Splash4 MOVLW 08h ;Each 10h represents 100mS MOVWF 0Ch Splash5 CALL Cell5 ;Display Cell 5 for 50mS DECFSZ 0Ch GOTO Splash5 RETURN Main MOVLW 40h MOVWF 1Dh Main1 BTFSC 05,2 ;Test for button A. NOT pressed line = LOW GOTO Main2 ;Pressed MOVF 1Dh,0 ;Not pressed XORLW 00 BTFSC 03,2 ;Is file 1D = 0? GOTO Main1 ;Yes DECF 1Dh,1 ;No. Decrement Debounce file GOTO Main1 Main2 MOVF 1Dh,0 ;Move file 1D to W XORLW 00 BTFSS 03,2 ;Is file 1D = 0? Is the zero-flag SET? GOTO Main1 ;No CALL Splash ;Yes GOTO Main END |  |
| --- | --- | --- |

|  | The block of numbers below is the HEX file for Experiment-23. Copy and paste it into a text program such as TEXTPAD or NOTEPAD and call it: Expt-23.hex. |
| --- | --- |
|  | :100000008316043085000030860083127328003088 :1000100091000030920008309300003094000030CE :1000200095003E28003091001C309200143093005F :100030001C309400003095003E283E309100223064 :10004000920022309300223094003E3095003E28EA :100050004130910041309200413093004130940092 :10006000413095003E280030910000309200003071 :10007000930000309400003095003E288514000065 :10008000851011088600512012088600512013089F :100090008600512014088600512015088600512042 :1000A00008009B0B51280030860005140000051045 :1000B000080050308C0007208C0B5B2810308C001F :1000C00012208C0B602810308C001D208C0B6528B2 :1000D00010308C0028208C0B6A2808308C003320CC :1000E0008C0B6F28080040309D00051D7D281D08E1 :1000F000003A031975289D0375281D08003A031D51 :0601000075285920732848 :00000001FF |

EXPERIMENT-24              
This is an extension of experiment 23. Press button A to start and stop SPLASH animation. Button A becomes a TOGGLE BUTTON - with START/STOP feature. 
It is important to label the flags so that you know exactly what they are indicating. 
The two flags in this experiment detect the same thing (button-press) but their position in the program makes them detect different features. 
Flag 1E,5 send s the micro to the SPLASH routine while 1E,4 detects when the button is NOT pressed so that the program can determine when a toggle action has occurred with the button. 
Creating a toggle feature takes a lot of thought as the instructions must be placed in the right places in the program to achieve the desired result.

|  | Experiment-24 for "5x7 Display" Project ;PIC16F84 and only F84 chip ;Press button A to START/STOP Splash animation Start ORG 0x00 BSF 03,5 ;Go to page1 to set-up the ports MOVLW 04h ;Put 04 into W MOVWF 05h ; to make RA2 input MOVLW 00h ;Put 00 into W MOVWF 06h ;and make port 6 (port B) all output BCF 03,5 ;Go to page0 for programming GOTO Main Cell1 MOVLW 00h ;place the 5 values into the 5 MOVWF 11h ; files to produce the first cell MOVLW 00h ; as shown above. MOVWF 12h MOVLW 08h MOVWF 13h MOVLW 00h MOVWF 14h MOVLW 00h MOVWF 15h GOTO Scan Cell2 MOVLW 00h ;place the 5 values into the 5 MOVWF 11h ; files to produce the second cell MOVLW 1Ch ; as shown above. MOVWF 12h MOVLW 14h MOVWF 13h MOVLW 1Ch MOVWF 14h MOVLW 00h MOVWF 15h GOTO Scan Cell3 MOVLW 3Eh ;place the 5 values into the 5 MOVWF 11h ; files to produce the third cell MOVLW 22h ; as shown above. MOVWF 12h MOVLW 22h MOVWF 13h MOVLW 22h MOVWF 14h MOVLW 3Eh MOVWF 15h GOTO Scan Cell4 MOVLW 41h ;place the 5 values into the 5 MOVWF 11h ; files to produce the fourth cell MOVLW 41h ; as shown above. MOVWF 12h MOVLW 41h MOVWF 13h MOVLW 41h MOVWF 14h MOVLW 41h MOVWF 15h GOTO Scan Cell5 MOVLW 00h ;place the 5 values into the 5 MOVWF 11h ; files to produce the fifth cell MOVLW 00h ; as shown above. MOVWF 12h MOVLW 00h MOVWF 13h MOVLW 00h MOVWF 14h MOVLW 00h MOVWF 15h GOTO Scan Scan BSF 05,1 ;Reset 4017 NOP BCF 05,1 MOVF 11h,0 ;Move file 11 into W MOVWF 06h CALL DelD MOVF 12h,0 ;Move file 12 into W MOVWF 06h CALL DelD MOVF 13h,0 ;Move file 13 into W MOVWF 06h CALL DelD MOVF 14h,0 ;Move file 14 into W MOVWF 06h CALL DelD MOVF 15h,0 ;Move file 15 into W MOVWF 06h CALL DelD RETURN DelD BTFSC 05,2 ;A pushed? Pushed = HIGH GOTO Del1 BCF 1E,4 ;Clear the button-pushed flag GOTO Del2 Del1 BSF 1E,4 ;Set the button-pushed flag Del2 DECFSZ 1Bh,1 GOTO Del2 MOVLW 00h ;Zero port B to prevent mirroring MOVWF 06 Clk BSF 05,0 ;Clock the 4017 NOP BCF 05,0 BTFSS 05,2 ;A pushed? RETURN ;A not pushed BTFSC 1E,4 ;Has the button been released? RETURN ;No. BCF 1E,5 ;Yes. Don't send Micro to SPLASH RETURN Splash MOVLW 50h ;Each 10h represents 100mS MOVWF 0Ch Splash1 CALL Cell1 ;Display Cell 1 for 1/2 sec DECFSZ 0Ch GOTO Splash1 MOVLW 10h ;Each 10h represents 100mS MOVWF 0Ch Splash2 CALL Cell2 ;Display Cell 2 for 100mS DECFSZ 0Ch GOTO Splash2 MOVLW 10h ;Each 10h represents 100mS MOVWF 0Ch Splash3 CALL Cell3 ;Display Cell 3 for 100mS DECFSZ 0Ch GOTO Splash3 MOVLW 10h ;Each 10h represents 100mS MOVWF 0Ch Splash4 CALL Cell4 ;Display Cell 4 for 100mS DECFSZ 0Ch GOTO Splash4 MOVLW 08h ;Each 10h represents 100mS MOVWF 0Ch Splash5 CALL Cell5 ;Display Cell 5 for 50mS DECFSZ 0Ch GOTO Splash5 RETURN Main BCF 1E,5 ;This flag sends micro to SPLASH BCF 1E,4 ;This flag detects button NOT pressed Main1 BTFSS 05,2 ;Is button A pushed? Pushed = HIGH GOTO Main2 ;Not pressed BSF 1E,5 ;Pressed. Set 1E,5 flag CALL Splash GOTO Main1 Main2 BTFSS 1E,5 GOTO Main1 CALL Splash GOTO Main1 END |  |
| --- | --- | --- |

|  | The block of numbers below is the HEX file for Experiment-24. Copy and paste it into a text program such as TEXTPAD or NOTEPAD and call it: Expt-24.hex. |
| --- | --- |
|  | :100000008316043085000030860083127D2800307E :1000100091000030920008309300003094000030CE :1000200095003E28003091001C309200143093005F :100030001C309400003095003E283E309100223064 :10004000920022309300223094003E3095003E28EA :100050004130910041309200413093004130940092 :10006000413095003E280030910000309200003071 :10007000930000309400003095003E288514000065 :10008000851011088600512012088600512013089F :100090008600512014088600512015088600512042 :1000A0000800051955281E1256281E169B0B5628A7 :1000B00000308600051400000510051D08001E1AFA :1000C00008009E12080050308C0007208C0B652819 :1000D00010308C0012208C0B6A2810308C001D20F0 :1000E0008C0B6F2810308C0028208C0B7428083063 :1000F0008C0033208C0B792808009E121E12051DDF :1001000084289E1663207F289E1E7F2863207F28D8 :00000001FF |

EXPERIMENT-25           "BULL'S EYE"
This is a simple "Hit the LED" game. The LEDs flash randomly on the screen and the player is required to press button B when the centre LED is illuminated.  When the "Bull's Eye" is hit, a siren sound will be produced from the piezo. 
The program has a number of interesting features. 
One of the biggest problems with computers is the generating of random numbers. Computers are not very good at doing this. In this program we need to produce random numbers for the LEDs on the screen, and every 6th to 8th LED needs to be the centre LED.  There are two ways to do this. One is a random number generator and the other is to produce a long table. If the table is long enough, the player will not be aware of any pattern or repetition.  This is the simplest approach as we need the centre LED illuminated at regular intervals. 
The second feature is the co-inciding of the button-press with the illuminated LED. This sounds complicated but it can be done with a simple sub-routine. To make sure the button is pressed ONLY when the LED is illuminated, the routine has to check the button is NOT PRESSED before the centre LED is lit.  
This program has only one cycle of operation. You are required to add more "Bull's Eye's" and make it more complex. The "Bull's Eye" section in the Main routine is identified. Simply copy the random LEDs sections (the sets of four instructions in the Main routine), then add another Bull's Eye section. Make sure the GOTO statement is anything other than "x" 
The Siren routine has been taken from "Library of Routines" with a couple of changes. 
The Delay routine has been taken from a previous experiment and the clocking of the "shift register" has been explained previously. 
Even though this program consists of over 100 lines of code, many are repetitions and other lines of code have been produced previously. 
The Delay routine determines the length of time the LED is illuminated on the screen. Three routines are provided: short, medium and long. 
"Row" sub-routine clocks the shift register (4017) to the desired row then the required LED is turned on. 
If you want to produce a more-complex game, the main program will become very large. The only alternative is to produce the LEDs via a random number generator. After every 6 - 10 random number, a "Bull's Eye" code is generated and in this way an infinite, random game will be created. 
For the moment we will stick to simple, "Linear Programming" and you can change and improve the "Bull's Eye" program.

|  | Experiment-25 for "5x7 Display" Project ;PIC16F84 and only F84 chip ;"Bull's Eye" Start ORG 0x00 BSF 03,5 ;Go to page1 for setting-up the ports MOVLW 08h ;Put 08 into W MOVWF 05h ; to make RA3 input MOVLW 00h ;Put 00 into W MOVWF 06h ;and make port 6 (port B) all output BCF 03,5 ;Go to page0 for programming BCF 1E,7 ;Clear the flag bit GOTO Main Delay1 MOVLW 01 MOVWF 1Ah Del1A DECFSZ 1Bh,1 GOTO Del1A DECFSZ 1Ch,1 GOTO Del1A DECFSZ 1Ah,1 GOTO Del1A RETURN Delay2 MOVLW 02 MOVWF 1Ah Del2A DECFSZ 1Bh,1 GOTO Del2A DECFSZ 1Ch,1 GOTO Del2A DECFSZ 1Ah,1 GOTO Del2A RETURN Delay3 MOVLW 03 MOVWF 1Ah Del3A DECFSZ 1Bh,1 GOTO Del3A DECFSZ 1Ch,1 GOTO Del3A DECFSZ 1Ah,1 GOTO Del3A RETURN Siren MOVLW 01h ; = 1 loop MOVWF 1Ch ; of Siren sound SirnD MOVLW 10h ;No of on-off cycles MOVWF 0Eh MOVWF 10h MOVLW 30h ;No of steps MOVWF 0Fh MOVLW 50h ;Determines frequency MOVWF 0Ch Repeat MOVF 0C,0 MOVWF 0D On BSF 06,7 DECFSZ 0D,1 GOTO On MOVWF 0D Off BCF 06,7 DECFSZ 0D,1 GOTO Off DECFSZ 10h,1 GOTO Repeat DECF 0C,1 INCF 0E,1 MOVF 0E,0 MOVWF 10h DECFSZ 0F,1 GOTO Repeat DECFSZ 1C,1 ;1 cycle of Siren sound GOTO SirnDel RETURN Row1 BSF 05,1 ;The row routine clocks the 4017 to the required row NOP BCF 05,1 Return Row2 BSF 05,1 NOP BCF 05,1 NOP BSF 05,0 NOP BCF 05,0 Return Row3 BSF 05,1 NOP BCF 05,1 NOP BSF 05,0 NOP BCF 05,0 NOP BSF 05,0 NOP BCF 05,0 Return Row4 BSF 05,1 NOP BCF 05,1 NOP BSF 05,0 NOP BCF 05,0 NOP BSF 05,0 NOP BCF 05,0 NOP BSF 05,0 NOP BCF 05,0 RETURN Row5 BSF 05,1 NOP BCF 05,1 NOP BSF 05,0 NOP BCF 05,0 NOP BSF 05,0 NOP BCF 05,0 NOP BSF 05,0 NOP BCF 05,0 NOP BSF 05,0 NOP BCF 05,0 RETURN Main CALL Row1 MOVLW 01 MOVWF 06 CALL Delay1 CALL Row5 MOVLW 08 MOVWF 06 CALL Delay2 CALL Row3 MOVLW 20 MOVWF 06 CALL Delay3 CALL Row4 MOVLW 10 MOVWF 06 CALL Delay1 CALL Row1 MOVLW 01 MOVWF 06 CALL Delay1 CALL Row3 MOVLW 20 MOVWF 06 CALL Delay1 CALL Row1 MOVLW 02 MOVWF 06 CALL Delay1 CALL Row2 MOVLW 04 MOVWF 06 CALL Delay1 BTFSC 05,3 ;Button B pressed? This is the "Bull's Eye" section BSF 1E,7 CALL Row3 MOVLW 08 MOVWF 06 CALL Delay1 BTFSS 05,3 GOTO x BTFSC 1E,7 GOTO x CALL Siren x CALL Row5 MOVLW 02 MOVWF 06 CALL Delay1 CALL Row4 MOVLW 08 MOVWF 06 CALL Delay3 CALL Row5 MOVLW 04 MOVWF 06 CALL Delay2 CALL Row5 MOVLW 20 MOVWF 06 CALL Delay2 CALL Row2 MOVLW 10 MOVWF 06 CALL Delay2 CALL Row3 MOVLW 40 MOVWF 06 CALL Delay1 CALL Row3 MOVLW 20 MOVWF 06 CALL Delay1 CALL Row2 MOVLW 40 MOVWF 06 CALL Delay3 BCF 1E,7 GOTO Main END |  |
| --- | --- | --- |

|  | The block of numbers below is the HEX file for Experiment-25. Copy and paste it into a text program such as TEXTPAD or NOTEPAD and call it: Expt-25.hex. |
| --- | --- |
|  | :100000008316083085000030860083129E137B28FB :1000100001309A009B0B0A289C0B0A289A0B0A288D :10002000080002309A009B0B13289C0B13289A0B94 :100030001328080003309A009B0B1C289C0B1C28DB :100040009A0B1C28080001309C0010308E00900094 :1000500030308F0050308C000C088D0086178D0BCF :100060002E288D0086138D0B3228900B2C288C03A4 :100070008E0A0E0890008F0B2C289C0B2528080058 :1000800085140000851008008514000085100514F3 :1000900000000510080085140000851000000514FC :1000A000000005100000051400000510080085146C :1000B0000000851000000514000005100000051464 :1000C000000005100000051400000510080085144C :1000D0000000851000000514000005100000051444 :1000E00000000510000005140000051000000514B4 :1000F000000005100800402001308600082067201D :100100000830860011204B20203086001A2057200E :1001100010308600082040200130860008204B2047 :10012000203086000820402002308600082044202D :1001300004308600082085199E174B200830860061 :100140000820851DA6289E1BA62823206720023094 :10015000860008205720083086001A2067200430C7 :1001600086001120672020308600112044201030A6 :10017000860011204B204030860008204B20203084 :10018000860008204420403086001A209E137B28D9 :1001900040200830860008205720103086001120AB :1001A0004B20043086001120672002308600112089 :1001B0004B2020308600082067200430860008206D :0201C0007B289A :00000001FF |

Go to the next page of experiments:    5x7 EXPERIMENTS: Page-10